PGTS Portable Gunnery training System users manual

Version 2.0

Microsoft Windows XP

Microsoft Windows 2000

Microsoft Windows 98

Prepared by:	C M Cooke

Date:			04th March 2003

# Change History

| Version | Date | Modification Details | Author |
| --- | --- | --- | --- |
| 2.0 | 04/03/03 | Initial document issue | CMC |

TABLE OF CONTENTS

1.	About PGTS	4

2.	How to use PGTS	5

3.	Ammunition	8

4.	Controls	9

5.	Crew	14

6.	 Debrief	15

7.	 Engagements	16

8.	Firing Modes	17

9.	Practices	18

10. Ranges	19

11.	Sighting Equipment	20

12.	Targetry	21

ABOUT PGTS

PGTS IS DESIGNED AS A TRAINING AID AND FAMILIARISATION SYSTEM FOR CHALLENGER 1 GUNNERY. IT IS WRITTEN IN VISUAL BASIC VERSION 5 BY A QUALIFIED CHALLENGER CREW COMMANDER. THIS VERSION IS AN EARLY RELEASE AND MAY THEREFORE CONTAIN SOME SMALL ERRORS.

HOW TO USE P.G.T.S.

P.G.T.S.  IS DESIGNED TO ACCURATELY SIMULATE CHALLENGER 1 GUNNERY IN ALL ASPECTS THEREFORE ONCE FAMILIAR WITH THE CONTROL SYSTEMS A TRAINED GUNNER SHOULD BE ABLE TO CARRY OUT ENGAGEMENTS AS PER RANGES. FOR THOSE UNFAMILIAR WITH CHALLENGER BELOW IS A WALK THRU FOR A BASIC ENGAGEMENT.

I. ON THE INTRO SCREEN SELECT START AND THIS WILL TAKE YOU TO THE OPTIONS SCREEN.

2. TAKE SOME TIME TO LOOK AROUND THIS SCREEN AND FAMILIARISE YOURSELF WITH THE VARIOUS OPTIONS.

3. SELECT THE FOLLOWING OPTIONS.

FROM THE OPTIONS FRAME

MODE				VISUAL

BURSTS				TWO

WIND-BIAS			NONE

TARGETRY 			AUTOMATIC

SYSTEM				FULL SYSTEM

ENGAGEMENTS		MIXED

TARGETS				6

TARGET TIMINGS		UP TILL DESTROYED

FROM THE CONTROLS FRAME

JOYSTICK				IF USING A JOYSTICK

MOUSE				FOR MOUSE CONTROL

KEYBOARD			FOR KEYBOARD CONTROL

ALL OTHER OPTIONS SHOULD BE LEFT AS THEY ARE.

4. IF USING A JOYSTICK CLICK ON "CALIBRATE JOYSTICK" AND FOLLOW THE INSTRUCTIONS GIVEN TO CALIBRATE THE JOYSTICK.

5. CLICK ON "START SIM" AND YOU WILL BE GIVEN A VEIW LOOKING THROUGH THE GUNNERS SIGHT.

6. YOU NOW NEED TO FAMILIARISE YOURSELF WITH THE CONTROL METHOD THAT YOU ARE USING. (SEE RELEVANT SECTION IF NECCASARY)

7. WHEN YOU ARE CONFIDENT WITH YOUR CONTROLS YOU SHOULD CLICK ON START AND THE ENGAGEMENTS WILL BEGIN.

8. THE FIRST THING YOU WILL HEAR IS THE CRY OF "ACTION!" FROM THE COMMANDER THIS GIVES YOU A FEW SECONDS WARNING BEFORE THE ENGAGEMENTS BEGIN"

9 FROM THIS POINT ONWARDS IT IS DOWN TO YOU AS THE GUNNER TO CARRY OUT THE CORRECT ENGAGEMENT DRILLS AS GIVEN IN THE FIRE ORDERS THAT YOU RECEIVE.

10. HAVING RECIEVED A NEW FIREORDER THE GUNNER WILL SELECT EITHER COAX OR MAIN AMMUNITION USING THE ARMAMENT SELECTO SWITCH.  IF USING MAIN ARMAMENT HE WILL THEN USE THE AMMUNITION SELECTOR SWITCH TO SELECT THE CORRECT NATURE OF AMMUNITION.

11. THE GUNNER WILL LAY THE MBS ONTO THE CENTRE OF THE TARGET WHEN HE IS HAPPY WITH HIS LAY HE WILL PRESS THE LASE SWITCH.

12. THE CORRECT RANGE TO THE TARGET SHOULD APPEAR IN THE LEFT EYEPEICE AND AN ELIPSE SHOULD APPEAR SNUGGLY AROUND THE TARGET

13. THE GUNNER WILL NOW PRESS THE AUTOLAY SWITCH AND KEEP IT PRESSED. THE ELIPSE WILL APPEAR AT THE CORRECT POINT ON THE GRATICULE PATTERN AND THE GUN WILL DRIVE UP UNTIL THE ELIPSE IS ONCE AGAIN SNUGGLY AROUND THE TARGET.

14. ONCE THIS HAS HAPPENED THE GUNNER WILL FIRE THE WEAPON USING THE FIRING SWITCH AND THE ROUND OR ROUNDS WILL BE FIRED AT THE TARGET. IF FIRING HESH THE AUTOLAYSWITCH MUST BE KEPT DEPRESSED IN ORDER TO ENSURE REVERSE AUTOLAY TAKES PLACE SO CORRECTIONS CAN BE MADE IF NECCASARY.

15. THE GUNNERS NEXT ACTIONS WILL DEPEND ON THE NATURE OF THE ENGAGEMENT AND WETHER OR NOT THE TARGET WAS HIT. THE OPTIONS ARE AS FOLLOWS.

16. FIN / DST ENGAGEMENT

ROUND HITS TARGET = TARGET WILL EXPLODE COMMANDER WILL ISSUE "TARGET STOP!"  USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON. ENGAGEMENT IS FINISHED WAIT FOR NEXT FIREORDER IF APPLICABLE.

ROUND MISSES TARGET =   USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON.THEN CARRY OUT DRILLS ONE TO FIVE AGAIN. REPEAT UNTIL TARGET IS DESTROYED OR DISAPEARS.

17. HESH ENGAGEMENT

ROUND HITS TARGET = TARGET WILL EXPLODE COMMANDER WILL ISSUE "TARGET STOP!"  USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON. ENGAGEMENT IS FINISHED WAIT FOR NEXT FIREORDER IF APPLICABLE.

ROUND MISSES TARGET =

1A. AFTER THE HESH ROUND HAS LANDED THE MBS IS PLACED ON THE FALL OF SHOT AND THE BUTTON IS PRESSED THE MBS IS THEN PLACED ONTO THE CENTER OF THE TARGET AND THE BUTTON IS PRESSED AGAIN.

2A. THE COMPUTER WILL THEN CALCULATE THE AIM OFF REQUIRED   TO HIT THE TARGET. THE GUNNER WILL THEN REDEMAND AUTOLAY (ENSURING HE KEEPS IT PRESSED) AND THE ELIPSE WILL MOVE TO THE CORRECTED POINT ON THE GRATICULE PATTERN.

3A. THE GUNNER CAN THEN FIRE THE WEAPON KEEPINGAUTOLAY PRESSED ALL THE TIME AFTER THE ROUND IS FIRED REVERSE AUTOLAY WILL TAKE PLACE. IF THE ROUND MISSES AGAIN REPEAT STEPS 1A TO 3A.

18. COAX ENGAGEMENTS

ROUNDS HITS TARGET = THE GUNNER WILL FIRE FURTHER BURSTS UNTIL THE TARGET EXPLODES COMMANDER WILL ISSUE "TARGET STOP!"  USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON. ENGAGEMENT IS FINISHED WAIT FOR NEXT FIREORDER IF APPLICABLE

ROUNDS MISS TARGET	=   THE GUNNER WILL OBSERVE THE FALL OF SHOT ON THEGRATICULE PATTERN AND PLACE THIS POINT ONTO THE CENTRE OF THE TARGET FIRING ANOTHER BURST. ONCE THE ROUNDS HIT THE TARGET THE GUNNER WILL FIRE FURTHER BURSTS UNTIL THE TARGET EXPLODES COMMANDER WILL ISSUE "TARGET STOP!"  USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON. ENGAGEMENT IS FINISHED WAIT FOR NEXT FIREORDER IF APPLICABLE

19 A SUUCESFULL ENGAGEMENT WILL SEE THE TARGET DESTROYED BEFORE IT DISAPEARS.

- ONCE ALL ENGAGEMENTS ARE COMPLETED YOU WILL BE TAKEN TO THE DEBREIF SCREEN WHERE YOU CAN CHECK ON THE RESULTS AND STANDARDS OF YOUR ENGAGEMENTS.

AMMUNITION

THE CHALLENGER CAN FIRE FOUR DIFFERENT TYPES OF MAIN ARMAMENT AMMUNITION THESE ARE AS FOLLOWS.

APFSDS (armour piercing fin stabilised discarding sabot)

REFERED TO AS FIN BY CREWS THIS IS THE MAIN "TANK KILLING" ROUND AND CONSISITS OF A TUNGSTEN DART ENCASED IN A METAL BOOT (sabot)  WHICH FLIES OFF WHEN THE ROUND LEAVES THE BARREL.

DST (discarding sabot trainer)

THIS IS A PRACTICE ROUND USED TO SIMULATE FIRING FIN. THOUGH ALIKE IN FLIGHT SPEED IT IS FAR LIGHTER, CHEAPER AND DOES CONSIDERABLY LESS DAMAGE.

HESH (high explosive squash head)

A LARGE HEAVY BULLET STYLE ROUND WITH AN EXPLOSIVE CONTENT USED AGAINST LIGHTLY ARMOURED VEHICLES AND GROUN TARGETS (trenches etc)

SH/P (squash head practice)

IDENTICAL IN SHAPE SIZE AND WEIGHT TO HESH IT DOES NOT CONTAIN ANY EXPLOSIVES AND IS THEREFORE USED IN PRACTICE ONLY.

IN ADDITION TO THIS THE CHALLENGER ALSO HAS TWO MACHINE GUNS ONE OF WHICH IS MOUNTED COAXIALLY; THIS MEANS IT FIRES ALONG THE SAME TRAJECTORY AS THE MAIN ARMAMENT.

ALL NATURES OF AMMUNITION HAVE A TRACE ELEMENT WHICH MEANS THEY PRODUCE A RED GLOW VISIBLE TO THE CREW WHEN THE ROUND IS FIRED. THIS ENABLES THE CREW TO TRACK THE ROUND AND APPLY AND CORRECTIONS NEEDED.

CONTROLS

THERE ARE FOUR DIFFERENT METHODS OF CONTROL FOR PGTS THESE ARE AS FOLLOWS:

MOUSE

IF MOUSE CONTROL IS SELECTED THEN A NUMBER OF BUTTONS WILL APPEAR UNDER THE GUNNERS SIGHT. THESE CONSIST OF FOUR DIRECTIONAL ARROWS, MAIN ARMAMENT SELECTION SWITCH, COAX SELECTION SWITCH, DE-SELECTER SWITCH, LAZE, AUTOLAY AND AMMUNITION SELCTION SWITCH. THESE CONSTITUTE THE MAIN GUNNERS CONTROLS FURTHER DETAILS CAN BE FOUND UNDER THAT HEADING

KEYBOARD

IF KEYBOARD IS SELECTED THE MOUSE CONTROLS WILL NOT APPEAR AND KEYBOARD BUTTONS WILL OPERATE THE GUNNERS CONTROLS AS FOLLOWS:

ELEVATE        	 	=    U

DEPRESS    		=    N

TRAV LEFT    		=    H

TRAV RIGHT 	 	=    J

SELECT MAIN		=    Q

DESELECT		=    W

SELECT COAX	 	=    E

LASE			=    A

AUTOLAY		=    S

FIRE				=    D

JOYSTICK

IF JOYSTICK IS SELECTED THEN YOU WILL BE PROMPED TO CALIBRATE IT BEFORE YOU BEGIN. THE SYSTEM CAN SUPPOERT EITHER TWO OR FOUR BUTTON JOYSTICKS AND THOSE CONTROLS NOT SUPPORTED BY THE JOYSTICK WILL STILL OPERATE THROUGH THE KEYBOARD THEREFORE WITH A KEYBOARD PROGRAMMABLE JOYSTICK ALL GUNNERS CONTROLS CAN BE OPERATED GIVING FAR MORE ACCURATE SIMULATION. JOYSTICK BUTTON CONTROLS ARE AS FOLLOWS:

BUTTON1 	=    FIRE

BUTTON2		=    AMM0

BUTTON3		=    LASE

BUTTON4		=    AUTOLAY

STAND

STILL UNDER DEVELOPMENT THE STAND WILL BE A REPLICA OF THE CHALLENGERS CONTROLS MOUNTED ON A LIGHT WEIGHT STAND. ALL CONTROLS WILL OPERATE AS FOR THE ACTUAL VEHICLE.

GUNNERY CONTROLS

DESCRIPTION

THE GUNNER OF A CHALLENGER HAS A LARGE NUMBER OF CONTROLS SO FOR THE SAKE OF SIMPLICITY ONLY THE IMPORTANT FIRING CONTROLS ARE INCLUDED IN THIS SIMULATOR.

THE MAIN CONTROLS THE GUNNER USES TO CARRY OUT ENGAGEMENTS ARE AS FOLLOWS:

ARMAMENT SELECTOR SWITCH

THIS IS A ROCKER SWITCH WITH THREE POSITIONS AND THESE ARE AS FOLLOWS:

COAX SELECTOR

THIS SELECTS THE COAXIAL MACHINE GUN AND DISABLES THE MAIN ARMAMENT THE CONTROLS FOR THIS ARE AS FOLLOWS.

MOUSE		=		COAX

KEYBOARD		=		Q

JOYSTICK		=		(PROGRAMME AS Q OR USE KEYBOARD)

MAIN ARMAMENT SELECTOR

THIS SELECTS THE MAIN ARMAMENT AND FIN AS THE DEFAULT AMMUNITION. THE CONTROLS FOR THIS ARE AS FOLLOWS.

MOUSE		=		MAIN

KEYBOARD		=		E

JOYSTICK		=		(PROGRAMME AS E OR USE KEYBOARD)

DESELECT

THIS DESELECTS ALL WEAPONS AND MUST BE DONE AFTER ALL ENGAGEMENTS THE CONTROLS FOR THIS ARE AS FOLLOWS.

MOUSE		=		OFF

KEYBOARD		=		W

JOYSTICK		=		(PROGRAMME AS W OR USE KEYBOARD)

AMMUNITION SELECTION SWITCH

THIS SWITCH HAS TWO FUNCTIONS THESE ARE AS FOLLOWS

SELECT AMMO NATURE

CLICKING ON THIS WILL CYCLE THROUGH THE DIFFERENT TYPES OF MAIN ARMAMENT AMMUNITION TO YOU FIND THE CORRECT ONE.

HESH CORRECTIONS

THE SWITCH WILL BE USED IN THIS MODE DURING HESH ENGAGEMENTS. AFTER THE HESH ROUND HAS LANDED THE MBS IS PLACED ON THE FALL OF SHOT AND THE BUTTON IS PRESSED THE MBS IS THEN PLACED ONTO THE CENTER OF THE TARGET AND THE BUTTON IS PRESSED AGAIN. THE COMPUTER WILL THEN CALCULATE THE AIM OFF REQUIRED   TO HIT THE TARGET. THIS WILL BE APPLIED NEXT TIME AUTOLAY IS DEMANDED.

LASE SWITCH

THIS SWITCH FIRES THE LASER WHICH CALCULATES THE RANGE TO THE TARGET AND PASSES THE INFORMATION TO THE IFCS WHICH USES IT TO CALCULATE THE CORRECT LAY OF THE GUN.

MOUSE		=		LASE

KEYBOARD		=		A

JOYSTICK		=		(PROGRAMME AS A OR BUTTON 2)

AUTOLAY

THIS SWITCH ACTUATES THE IFCS WICH THEN CARRIES OUT A CONTROLLED LAY OF THE GUN TO TRY AN INSURE A HIT ON TARGET USING ALL AVAILABLE INFORMATION.

MOUSE		=		AUTO

KEYBOARD		=		S

JOYSTICK		=		(PROGRAMME AS S OR BUTTON 3)

FIRE

THIS SWITCH WILL FIRE THE SELECTED WEAPON EITHER MAIN ARMAMENT OR COAX MACHINE GUN.

MOUSE		=		FIRE

KEYBOARD		=		D

JOYSTICK		=		(PROGRAMME AS D OR BUTTON 1)

USING THE GUNNERS CONTROLS

THE NORMAL ORDER OF USE FOR AN ENGAGEMENT IS AS FOLLOWS.

1. HAVING RECIEVED A NEW FIREORDER THE GUNNER WILL SELECT EITHER COAX OR MAIN AMMUNITION USING THE ARMAMENT SELECTO SWITCH.  IF USING MAIN ARMAMENT HE WILL THEN USE THE AMMUNITION SELECTOR SWITCH TO SELECT THE CORRECT NATURE OF AMMUNITION.

2. THE GUNNER WILL LAY THE MBS ONTO THE CENTRE OF THE TARGET WHEN HE IS HAPPY WITH HIS LAY HE WILL PRESS THE LASE SWITCH.

3. THE CORRECT RANGE TO THE TARGET SHOULD APPEAR IN THE LEFT EYEPEICE AND AN ELIPSE SHOULD APPEAR SNUGGLY AROUND THE TARGET

4. THE GUNNER WILL NOW PRESS THE AUTOLAY SWITCH AND KEEP IT PRESSED. THE ELIPSE WILL APPEAR AT THE CORRECT POINT ON THE GRATICULE PATTERN AND THE GUN WILL DRIVE UP UNTIL THE ELIPSE IS ONCE AGAIN SNUGGLY AROUND THE TARGET.

5. ONCE THIS HAS HAPPENED THE GUNNER WILL FIRE THE WEAPON USING THE FIRING SWITCH AND THE ROUND OR ROUNDS WILL BE FIRED AT THE TARGET. IF FIRING HESH THE AUTOLAYSWITCH MUST BE KEPT DEPRESSED IN ORDER TO ENSURE REVERSE AUTOLAY TAKES PLACE SO CORRECTIONS CAN BE MADE IF NECCASARY.

6. THE GUNNERS NEXT ACTIONS WILL DEPEND ON THE NATURE OF THE ENGAGEMENT AND WETHER OR NOT THE TARGET WAS HIT. THE OPTIONS ARE AS FOLLOWS.

FIN / DST ENGAGEMENT

ROUND HITS TARGET = TARGET WILL EXPLODE COMMANDER WILL ISSUE "TARGET STOP!"  USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON. ENGAGEMENT IS FINISHED WAIT FOR NEXT FIREORDER IF APPLICABLE.

ROUND MISSES TARGET =   USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON.THEN CARRY OUT DRILLS ONE TO FIVE AGAIN. REPEAT UNTIL TARGET IS DESTROYED OR DISAPEARS.

HESH ENGAGEMENT

ROUND HITS TARGET = TARGET WILL EXPLODE COMMANDER WILL ISSUE "TARGET STOP!"  USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON. ENGAGEMENT IS FINISHED WAIT FOR NEXT FIREORDER IF APPLICABLE.

ROUND MISSES TARGET =

1A. AFTER THE HESH ROUND HAS LANDED THE MBS IS PLACED ON THE FALL OF SHOT AND THE BUTTON IS PRESSED THE MBS IS THEN PLACED ONTO THE CENTER OF THE TARGET AND THE BUTTON IS PRESSED AGAIN.

2A. THE COMPUTER WILL THEN CALCULATE THE AIM OFF REQUIRED   TO HIT THE TARGET. THE GUNNER WILL THEN REDEMAND AUTOLAY (ENSURING HE KEEPS IT PRESSED) AND THE ELIPSE WILL MOVE TO THE CORRECTED POINT ON THE GRATICULE PATTERN.

3A. THE GUNNER CAN THEN FIRE THE WEAPON KEEPINGAUTOLAY PRESSED ALL THE TIME AFTER THE ROUND IS FIRED REVERSE AUTOLAY WILL TAKE PLACE. IF THE ROUND MISSES AGAIN REPEAT STEPS 1A TO 3A.

COAX ENGAGEMENTS

ROUNDS HITS TARGET = THE GUNNER WILL FIRE FURTHER BURSTS UNTIL THE TARGET EXPLODES COMMANDER WILL ISSUE "TARGET STOP!"  USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON. ENGAGEMENT IS FINISHED WAIT FOR NEXT FIREORDER IF APPLICABLE

ROUNDS MISS TARGET	=   THE GUNNER WILL OBSERVE THE FALL OF SHOT ON THEGRATICULE PATTERN AND PLACE THIS POINT ONTO THE CENTRE OF THE TARGET FIRING ANOTHER BURST. ONCE THE ROUNDS HIT THE TARGET THE GUNNER WILL FIRE FURTHER BURSTS UNTIL THE TARGET EXPLODES COMMANDER WILL ISSUE "TARGET STOP!"  USE ARMAMENT SELECTOR SWITCH TO DESELECT WEAPON. ENGAGEMENT IS FINISHED WAIT FOR NEXT FIREORDER IF APPLICABLE

CREW

THE CREW OF A CHALLENGER CONSISTS OF FOUR PEOPLE THE COMMANDER, GUNNER, LOADER, AND DRIVER

COMMANDER

THE COMMANDER IS IN CHARGE OF THE TANK HE ISSUES THE FIREORDER AND POSITIONS THE GUN IN THE TARGET AREA BFORE THE GUNNER TAKES CONTROL

GUNNER

THE GUNNER IS RESPOSIBLE FOR THE CORRECTLAY OF THE GUN ONTO THE TARGET. HE CARRIES OUT THE CORRECT DRILLS AS PER THE COMMANDERS FIREORDER.

LOADER

THE LOADER WILL LOAD THE CORRECT NATURE OF AMMUNITION AS INDICATED BY THE COMMANDER IN HIS FIREORDER.

DEBRIEF

THE DEBREIFING IS USED TO ASSES THE GUNNERS PERFORMANCE DURING THE PRACTICE AND CONTAINS THE FOLLOWING INFORMATION ON EACH ENGAGEMENT.

RANGE TO TARGET

IF TARGET WAS DESTROYED OR NOT

AMMUNITION USED

NUMBER OF ROUNDS FIRED

TIME TAKEN FOR ENGAGEMENT

ADDITIONALLY THE DEBRIEF SCREEN SHOWS A GRAPHICAL REPRESENTATION OF THE LAY OF THE GUN WHEN IT WAS FIRED.

IF IT WAS A THERMAL PRACTICE THEN THE APPROPRIATE THERMAL SYMBOLOGY WILL BE SHOWN OTHERWISE THE GRATICULE PATTERN WILL BE PLACED OVER THE APPROPRIATE TARGET AS IT WAS WHEN THE WEAPON WAS FIRED.

THE SMALL NUMBERED TARGET SYMBOLS ON THE LEFT OF THE SCREEN REPRESENT THE VARIOUS ENGAGEMENTS FIRED. BY CLICKING ON ONE OF THESE YOU WILL SELECT ALL THE RELEVANT DETAILS.

IF MORE THAN ONE ROUND WAS FIRED THEN THE RELEVANT NUMBER OF BUTTONS WILL APPEAR UNDER THE ROUNDS FIRED COUNTER. CLICKING ON THESE BUTTONS WILL SHOW THE FINAL LAY OF THE WEAPON FOR THAT PARTICULAR ROUND.

USING THE CONTROLS AT THE TOP OF THE SCREEN YOU CAN SAVE DEBREIFINGS AND RE ASSESS OR COMPARE THEM AT A LATER DATE.

THE GUNNER CAN ALSO GET A PRINTOUT DISPLAYING THE STATISTICS OF ANY PRACTICE.

ENGAGEMENT

DESCRIPTION

AN ENGAGEMENT IS THE SEQUENCE OF EVENTS INVOVED IN DESTROYING A TARGET IT BEGINS WITH A FIREORDER AND IS COMPLETED WHEN THE TARGET IS EITHER DESTROYED OR WITHDRAWS. THE SEQUENCE OF AN ENGAGEMENT IS AS FOLLOWS.

1. THE COMMANDER LAYS THE GUNNER ON OR NEAR THE TARGET AS HE ISSUES A FIREORDER.

2. THE GUNNER LAYS THE GUN ONTO THE TARGET AND CARRIES OUT THE APPROPRIATE DRILLS TO DESTROY THE TARGET.

ENGAGEMENTS

TYPES OF ENGAGEMENT

THERE ARE FOUR DIFFERENT TYPES OF ENGAGEMENTS THESE ARE AS FOLLOWS.

| NATURE OF AMMO | DISCRIPTION | REMARKS |
| --- | --- | --- |
| COAX | 7.62 co-axial machine gun | Used against soft skinned vehicles and men targets |
| FIN/DST | DST = DISCARDING SABOT TRAINER ROUND FIN = ARMOUR PIERCING FIN STABILISED DISCARDING SABOT ROUND | Used against other tanks |
| HESH | HESH = HIGH EXPLOSIVES SQUASH HEAD | USED AGAINST LIGHTLY ARMOURED VEHICLES AND AREA TARGETS |

NUMBER OF ENGAGEMENTS

THE NUMBER OF ENGAGEMENTS MUST BE SELECTED ON THE SETUP SCREEN. THIS DICTATES HOW MANY TARGETS WILL APPEAR DURING THE PRACTICE THE VARIOUS OPTIONS ARE.

6, 9, 12, 15,

THESE ARE SELECTED ON THE TARGETS OPTION,

FIRING MODES (THERMAL AND VISUAL

MODE

THERE ARE TWO DIFFERENT TYPES OF MODE IN THE CHALLENGER THESE ARE VISUAL WHICH IS THE NORMAL DAY TIME SIGHTING SYSTEM, AND THERMAL WHICH IS USED IN POOR CONDITIONS AND AT NIGHT.

THESE TWO MODES CAN BE USED AS A FULL SYSTEM IF ALL COMPUTERS AND SENSORS ARE OPERATING OR REVERSIONARY MODES (Rev Modes).IF SOME ELEMENTS OF THE IFCS ARE NOT WORKING.

VISUAL

VISUAL IS THE NORMAL FIRING MODE FOR THE VEHICLE.IN THIS MODE YOU SEE THE VEIW OF THE RANGE THROUGHT THE GUNNERS DAY SIGHTS RIGHT EYEPIECE. YOU GET TARGET RANGE, AMMUNITION SELECTION, AND LASER CHARGE STATUS THROUGH THE LEFT EYEPIECE.

THERMAL

WHEN CONDUCTING THERMAL ENGAGEMENTS THE GUNNER USES A VEIWER WHICH SEES THE HEAT GIVEN OF BY OBJECTS RATHER THAN THE LIGHT. THIS ALLOWS THE TANK TO FIGHT AT NIGHT AND IN ADVERSE WEATHER CONDITIONS. ALLTHOUGH THE VEIW IS DIFFERENT THE CONTROLS AND GUNNERY METHODS ARE ALL BASICALLY THE SAME.

PRACTICES

DESCRIPTION

A PRACTICE IS A SERIES OF ENGAGEMENTS THAT THE GUNNERCONDUCTS UNDER THE CONTROL OF THE COMMANDER

CREATING

PRACTICES CAN BE SELECTED BY CHANGING THE VARIOUS OPTIONS IN THE PRACTICE EDITOR (number of targets, type of ammo, timings etc.). ONCE YOU HAVE SELECTED ALL THE DESIRED OPTIONS (you can check them in the practice description box) THEY CAN THEN BE SAVED TO A NAME OF YOUR CHOICE (see save practice).

OPENING

TO SELECT A PREVIOUSLY SAVED PRACTICE CLICK ON OPEN PRACTICE. A DIALOG BOX WILL BE DISPLAYED SHOWING ALL AVAILABLE PRACTICES ONCE YOU HAVE CHOSEN THE PRACTICE YOU REQUIRE CLICK ON OPEN AND THE DETAILS OF THE PRACTICE WILL BE DISPLAYED IN THE DETAILS BOX OF THE PRACTICES FRAME.

SAVING

TO SAVE A PRACTICE YOU HAVE CREATED ENSURE THE DETAILS OF THE PRACTICE (as shown in the details box) ARE CORRECT. CLICK ON SAVE PRACTICE IN THE SETUP SCREEN COMMANDS. A DIALOG BOX WILL APPEAR. TYPE IN THE NAME YOU WISH TO GIVE THE PRACTICE THEN CLICK ON SAVE.

CLOSING

TO CLOSE THE CURRENT PRACTICE AND RETURN TO THE DEFAULT CLICK ON THE STOP SIGN IN THE PRACTICES FRAME. THE DETAILS OF THE DEFAULT PRACTICE WILL THEN BE DISPLAYED IB THE DETAILS BOX OF THE PRACTICES FRAME.

PRE PROGRAMMED PRACTICES

THE COMPUTER HAS A NUMBER OF PRE PROGRAMMED PRACTICES THESE ARE SELECTED ON THE SETUP SCREEN BY CLICKING ON THE NUMBERED TANKS IN THE PRACTICES FRAME.

THE DETAILS OF ALL THESE PRACTICES ARE GIVEN IN THE DETAILS BOX ON THE PRACTICES FRAME BUT CAN BE ALTERED BY SELECTING OTHER OPTIONS; THESE CHANGED OPTIONS WILL THEN BE DISPLAYED IN THE DETAILS BOX.

RANGES

DESCRIPTION

RANGES ARE AREA`S OF LAND USED BY THE ARMY FOR TRAINING TANK CREWS TARGETS MADE FROM WOOD, CLOTH AND OLD VEHICLES ARE POSITIONED BETWEEN  500 & 5000 METRES FROM THE FIRING POINT.

THE RANGE PICTURES USED IN THIS PROGRAMME ARE ALL ACTUAL ARMY RANGES AND ARE COMPILED FROM STANDARD BITMAP PICTURES.

LOADING

TO SELECT AND LOAD A DIFFERENT RANGE CHOOSE LOAD RANGE FROM THE RANGE OPTIONS ON THE PRACTICE EDITOR SCREEN. THIS WILL DISPLAY A LIST OF ALL AVAILABLE RANGES.  CLICK ON THE DESIRED RANGE TO SELECT IT THEN CLICK ON OPEN. THE NAME OF THE SELECTED RANGE WILL BE DISPLAYED.

CREATING

1. ON THE SETUP SCREEN CHOOSE EDIT RANGE THIS WILL TAKE YOU TO THE RANGE EDITOR SCREEN.ON THIS SCREEN SELECT NEWRANGE

2. YOU WILL THEN BE ASKED TO CHOOSE A NAME FOR THE RANGE AND SELECT EITHER A VISUAL OR A THERMAL RANGE.

3. HAVING DONE THIS YOU WILL BE PRESENTED WITH A LIST OF AVAILABLE RANGE PICTURES SELECT THE DESIRED PICTURE BY CLICKING ON IT AND THEN CLICKING ON LOAD.

4. THE VEIW OF THE CHOSEN RANGE WILL BE SHOWN IN THE BOTTOM HALF OF THE SCREEN WITH THE VEIW OF THE TARGET THROUGH THE SIGHT IN THE TOP HALF.

5.ON THE TOP RIGHT OF THE SCREEN IS THE TARGET SELECTOR THERE ARE TWENTY TARGETS FOR EACH RANGE SELECT A TARGET NUMBER BY CLICKING ON IT (all targets must be placed).

6. ONCE YOU HAVE SELECTED A TARGET USE THE ARROW BUTTONS TO MOVE THE RED CIRCLE IN THE MAIN WINDOW THIS INDICATES WHERE THE TARGET WILL APPEAR DURING THE ENGAGEMENTS.

7. WHEN YOU ARE HAPPY WITH THE TARGETS LOCATION CLICK ON SAVE TARGET. THE TARGETS LOCATION AND RANGE ARE SHOWN IN THE INFORMATION BOX ON THE LEFT OF THE SCREEN IF YOU ARE UNHAPPY WITH THE GIVEN RANGE TO THE TARGET YOU CAN ENTER A MANUAL RANGE BY CLICKING ON TARGET RANGE AND ENTERING THE RANGE YOU DESIRE.

8. YOU SHOULD THEN REPEAT STEPS 5 TO 7 FOR ALL THE TARGET NUMBERS.

9. TO CHANGE A TARGET CLICK ON THE TARGET YOU WISH TO CHANGE AND A LIST OF ALL AVAILABLE TARGETS WILL APPEAR CLICK ON THE TARGET YOU WISH TO USE AND THEN CLICK ON LOAD.

SAVING

ONCE YOU HAVE COMPLETED YOUR RANGE CLICK ON SAVE RANGE FROM THE MENU BUTTONS AT THE TOP OF THE SCREEN. YOU WILL BE PRESENTED WITH A SAVE DIALOG BOX. ENTER YOUR CHOSEN NAME FOR THE RANGE AND CLICK ON SAVE.

EDITING

1. CLICK ON OPEN RANGE AND SELECT THE RANGE YOU WISH TO EDIT BY CLICKING ON IT AND THEN CLICKING ON LOAD.

2. YOU SHOULD THEN CARRY OUT STEPS 4 TO 9 OF CREATING A RANGE (Note when saving this range you should choose a different name or the original range settings will be overwritten

SIGHTING EQUIPTMENT AND STATUS

GUNNERS DAY SIGHT

THIS IS WHERE THE GUNNER OBSERVES AND ENGAGES THE TARGETS DURING NORMAL VISUALENGAGEMENTS. THROUGH THE RIGHT EYEPEICE HE CAN SEE DOWN RANGE AND THE TARGET, ON TOP OF THIS IS DISPLAYED THE GRATICULE PATTERN. THROUGH THE LEFT EYEPEICE HE HAS THE TELL BACK INFORMATION (AMMUNITION SELECTION, RANGE, LASER STATUS ETC).

THERMAL VEIWERS

THESE ARE THE SCREENS WHICH SHOW THE COMMANDER AND GUNNER THE THERMALPICTURE OF THE RANGE AND TARGET AS WELL AS OTHER INFORMATION SUCH AS AMMUNITION & LASED RANGE.

SYSTEM STATUS

THE SYSTEM STATUS INDICATES THE CONDITION OF THE TANKS IFCS (gunnery computer) AND CAN BE SET TO FOUR DIFFERENT SETTINGS THESE ARE.

FULL

THIS IS THE NORMAL OPERATING STATUS FOR THE TANK IT MEANS THAT ALL SYSTEMS ARE WORKING BOTH THERMAL AND VISUAL.

REV - MODES (reversionary modes)

THESE MODES ARE USED WHEN THE TANK HAS A PROBLEM WITH THE VARIOUS ASPECTS OF THE IFCS SYSTEM.

LASE ONLY			COMPUTER NOT WORKING BUT LASER IS

TLS ONLY			NEITHER LASER OR COMPUTER ARE 						WORKING

SELF DEGRADING		SYSTEM WILL GRADUALLY DEGRADE  					FROM FULL TO TLS ONLY

DEPENDING ON THE STATUS OF THE SYSTEM THE DRILLS AND FIREORDER WILL CHANGE.

TARGETRY

THERE ARE THREE DIFFERENT TYPES OF TARGETS AND EACH TYPE SHOULD BE ENGAGED BY A DIFFERENT NATURE OF AMMUNTION. THE VARIOUS TYPES ARE AS FOLLOWS

COAX TARGETS

THESE ARE SOFT SKINNED VEHICLES (LANDROVER TYPE) AND DISMOUNTED INFANTRY.

HESH TARGETS

THESE ARE LIGHTLY ARMOURED VEHICLES (APCs AND BMPs etc) AND INFANTRY IN TRENCHES.

FIN / DST TARGETS

THESE ARE OTHER TANK TARGETS (T72 T80 ETC).

CREATING TARGETS

ALL TARGETS ARE 64 * 64 256 COLOUR ICONS .TARGETS CAN BE CREATED USING ANY ICON CREATING SOFTWARE AND THEN SAVING THEM IN THE PGTS DIRECTORY.

CHANGING TARGETS

TARGETS CAN BE CHANGED USING THE RANGE EDITOR. SEE ALSO CREATING RANGES AND EDITING RANGES